## Workflow for selecting field stream reaches for EoA modeling with road access

GIS layers needed:

- Target basin polygons

- Create subbasins (NetMap tool), or find comparable HUC14 polygons for the target basins.

- Roads with access designation

- Stream layer

- Verified fish field points (ADFG, etc.)

- Snap field points to stream within target basins.  With field points, designate reaches as fish verified or unverified (upstream/downstream).  Cross reference with ADFG Anadromous Waters stream layer.  Be careful here, some ADFG points may not be field verified.

- Select reaches that fall within target basins. Classify as EoA.

- Filter roads with open access

- Create an 805-m (~ ½ mile) road buffer for open roads only

- Select all reaches designated as unverified that fall within the 805-m road buffer.

  - This will reduce hiking distance to make sure a study reach/subbasin is completed in a day.

  - Create new field (e.g., study_rch) and designate 1 = yes, 0 = no.

- Select subbasins with reaches designated as a study reach = 1.

- Import the reach and subbasin layers into R for GRTS spatial random sampling.

Notes:

If there are enough current Kenai field points (> 50), its probably worth the time to develop a preliminary model based on this information.  The results from this preliminary model can then be validated with the final model created with the new field data.